Hand Gesture Volume Controller (Arduino + Python)

Control your laptop’s volume with simple hand gestures using an ultrasonic sensor, Arduino, and Python.
This project uses Python as an interface between Arduino and your OS.

Project Overview

Goal:

- Move your hand closer to increase volume

- Move your hand away to decrease volume

Components:

- Arduino Uno / Nano

- HC-SR04 Ultrasonic Sensor

- Jumper wires

- USB cable to connect Arduino to laptop

Software:

- Arduino IDE

- Python 3

- Python libraries: pyserial, pyautogui

Hardware Wiring

HC-SR04 → Arduino

| Sensor Pin | Arduino Pin |
| --- | --- |
| VCC | 5V |
| GND | GND |
| TRIG | D9 |
| ECHO | D10 |

- TRIG = output (Arduino sends signal)

- ECHO = input (Arduino receives echo)

Arduino IDE Code

Upload this code to your Arduino:

#define trigPin 9

#define echoPin 10

long duration;

int distance;

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

Serial.begin(9600);

}

void loop() {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.034 / 2;

if (distance < 10) {

Serial.println("UP");

delay(500);

}

else if (distance > 20 && distance < 50) {

Serial.println("DOWN");

delay(500);

}

}

Explanation:

- Measures distance using ultrasonic sensor

- Sends "UP" if hand is close (<10 cm)

- Sends "DOWN" if hand is far (>20 cm, <50 cm)

- Communicates via Serial to Python

Python Script (Interface)

1️⃣ Install Python Libraries

pip install pyserial pyautogui

2️⃣ Python Code

Save as volume_control.py:

import serial

import time

import pyautogui

# Set the correct COM port for your Arduino

ser = serial.Serial('COM7', 9600)

print("Listening for hand gestures... ✋")

while True:

if ser.in_waiting:

command = ser.readline().decode().strip()

if command == "UP":

pyautogui.press("volumeup")

print("Volume UP 🔊")

elif command == "DOWN":

pyautogui.press("volumedown")

print("Volume DOWN 🔉")

time.sleep(0.2)

Explanation:

- Reads commands from Arduino via Serial

- pyautogui simulates keypresses

- "UP" → Volume Up key

- "DOWN" → Volume Down key

- time.sleep(0.2) prevents spamming

Interface Flow

- Hand gesture → ultrasonic sensor detects distance

- Arduino calculates distance → sends "UP" or "DOWN" via Serial

- Python script reads Serial data

- Python translates "UP" / "DOWN" → keyboard events using pyautogui

- Laptop OS receives keyboard input → changes volume

Visualized as a flow:

[Your Hand]

↓ (ultrasonic waves)

[HC-SR04 Sensor]

↓ (distance measured)

[Arduino]

↓ (Serial: "UP"/"DOWN")

[Python Script]

↓ (keyboard press simulated)

[Laptop OS]

↓ (volume changes)